High Performance Computing

System Administrator

CENTRE FOR DEVELOPMENT OF ADVANCED COMPUTING PUNE

CASE STUDY

Submitted By:

Suraj Bhujbal 		(230340127012)

Nitya Vats	 	(230340127004)

Siddhesh Kachkure 	(230340127010)

Indrajeet Kumar 	(230340127036)

AIM

Build a Two node Stateless HPC-Cluster using Warewulf with PBS, Ganglia, and LDAP

Required Machine: Centos7 Virtual Machine

Specification: 8 GB RAM, 4 Processor, 100 HDD

Network Adapter: NAT: 192.168.44.172, Host-Only: 10.0.0.129

WareWulf

On Master

# systemctl stop firewalld.service

# systemctl disable firewalld.service

# hostnamectl set-hostname master

# vi /etc/selinux/config

SELINUX=disabled

Warewulf Installation

# yum install -y https://repo.ctrliq.com/rhel/8/ciq-release.rpm

# yum install -y warewulf

# vim /etc/warewulf/warewulf.conf

WW_INTERNAL: 43

ipaddr: 10.0.0.129  	#(host ip address of master node)

netmask: 255.0.0.0

network: 10.0.0.0

warewulf:

port: 9873

secure: false

update interval: 60

autobuild overlays: true

host overlay: true

syslog: false

datastore: ""

dhcp:

enabled: true

template: default

range start: 10.0.0.1

range end: 10.0.0.100

systemd name: dhcpd

tftp:

enabled: true

tftproot: ""

systemd name: tftp

nfs:

enabled: true

export paths:

- path: /home

export options: rw,sync

mount options: defaults

mount: true

- path: /opt

export options: ro,sync,no_root_squash

mount options: defaults

mount: false

systemd name: nfs-server

# wwctl configure --all

# systemctl status dhcpd

# systemctl status tftp

# systemctl status nfs-server

# systemctl start warewulfd && systemctl enable warewulfd && systemctl status warewulfd

# wwctl server status

# more /var/log/warewulfd.log

# wwctl container import docker://ghcr.io/hpcng/warewulf-centos:7 centos-7

# wwctl container list

# wwctl container exec centos-7 /bin/bash

# [centos-7] Warewulf> exit

# wwctl container  list

# wwctl profile list -a

# wwctl container import docker://registry.opensuse.org/science/warewulf/tumbleweed/containerfile/kernel:latest tw

# wwctl kernel import -DC tw

Now adding Node

# wwctl node add n001 -I 10.0.0.2

# wwctl node set --container centos-7 n001

# wwctl node set --kerneloverride tw n001

# wwctl kernel list

# wwctl node set --netdev ens36 --hwaddr 00:0C:29:1E:E3:5D --ipaddr 10.0.0.2 --netmask 255.255.255.0 cn001

Now restart booted Node

Here we have successfully booted 2 nodes by Warewulf provisioning tool

Open LDAP

On Master

# systemctl stop firewalld && systemctl disable firewalld && systemctl status firewalld

# vi /etc/selinux/config

SELINUX=disabled

# vi /etc/hosts

Add hosts ip address and their hostname in the file

# yum -y install openldap-servers openldap-clients

# cp /usr/share/openldap-servers/DB_CONFIG.example /var/lib/ldap/DB_CONFIG

# systemctl start slapd && systemctl enable slapd && systemctl status slapd

# slappasswd

{SSHA}Vn+t6j5XSFmG9X1QIPFcDorZYF50LnS3

# vi chrootpw.ldif

# ldapadd -Y EXTERNAL -H ldapi:/// -f chrootpw.ldif

# ldapadd -Y EXTERNAL -H ldapi:/// -f /etc/openldap/schema/cosine.ldif

# ldapadd -Y EXTERNAL -H ldapi:/// -f /etc/openldap/schema/nis.ldif

# ldapadd -Y EXTERNAL -H ldapi:/// -f /etc/openldap/schema/inetorgperson.ldif

# vi chdomain.ldif

# ldapmodify -Y EXTERNAL -H ldapi:/// -f chdomain.ldif

# vi basedomain.ldif

# ldapadd -x -D cn=Manager,dc=cdac,dc=in -W -f basedomain.ldif

# vi ldapuser.ldif

’

# ldapadd -x -D cn=Manager,dc=cdac,dc=in -W -f ldapuser.ldif

On Client

# systemctl stop firewalld && systemctl disable firewalld && systemctl status firewalld

# vi /etc/selinux/config

SELINUX=disabled

# yum -y install openldap-clients nss-pam-ldapd

# authconfig --enableldap --enableldapauth --ldapserver=master --ldapbasedn="dc=cdac,dc=in" --enablemkhomedir --update

# systemctl restart nslcd && systemctl status nslcd

# ldapsearch -x

# cat /etc/nslcd.conf

# cat /etc/nsswitch.conf

# getent passwd

# ssh hpcsa@n001

Ganglia

On Master

# systemctl stop firewalld && systemctl disable firewalld && systemctl status firewalld

# vi /etc/selinux/config

SELINUX=disabled

# yum install http://build.openhpc.community/OpenHPC:/1.3/CentOS_7/x86_64/ohpc-release-1.3-1.el7.x86_64.rpm

# yum install ohpc-ganglia -y

# vim /etc/ganglia/gmetad.conf

# systemctl start gmetad.service gmond.service httpd.service && systemctl status gmetad.service gmond.service httpd.service && systemctl enable gmetad.service gmond.service httpd.service

# vi /etc/ganglia/gmond.conf

enter cluster name

enter master ip

comment these lines

# systemctl restart gmond && systemctl status gmond

On Client

# yum install http://build.openhpc.community/OpenHPC:/1.3/CentOS_7/x86_64/ohpc-release-1.3-1.el7.x86_64.rpm -y

# yum install ganglia-gmond-ohpc -y

# vi /etc/ganglia/gmond.conf

# systemctl restart gmond && systemctl status gmond

Now open web browser and enter ip address of master/ganglia/

PBS

On Master

# systemctl stop firewalld && systemctl disable firewalld && systemctl status firewalld

# vi /etc/selinux/config

SELINUX=disabled

# vi /etc/hosts

Add hosts ip address and their hostname in the file

# ssh-keygen

# ssh-copy-id root@n001

# ssh-copy-id root@n002

# yum install git -y

# git clone https://github.com/openpbs/openpbs.git

# yum group install "Development Tools"

# cd openpbs

# mv /root/openpbs/ /root/openpbs-23.06.06

# mkdir -p /root/rpmbuild/SOURCES

# tar -cvf /root/rpmbuild/SOURCES/openpbs-23.06.06.tar.gz openpbs-23.06.06

# cd openpbs-23.06.06/

# yum install libtool-ltdl-devel hwloc-devel libXt-devel libedit-devel libical-devel ncurses-devel postgresql-devel  postgresql-contrib python3-devel tcl-devel tk-devel zlib-devel expat-devel openssl-devel -y

# rpmbuild -ba openpbs.spec

# cd

# cd rpmbuild/RPMS/x86_64/

# yum install openpbs-server-23.06.06-0.x86_64.rpm -y

# chmod 4755 /opt/pbs/sbin/pbs_iff  /opt/pbs/sbin/pbs_rcp

# systemctl start pbs.service && systemctl enable pbs.service

# . /etc/profile.d/pbs.sh

# qstat -B

On Node 1

# yum install git -y

# git clone https://github.com/openpbs/openpbs.git

# cd openpbs/

# yum install autoconf autoconf-2.69 automake libtool -y

# yum install libtool-ltdl-devel hwloc-devel libXt-devel libedit-devel libical-devel ncurses-devel postgresql-devel  postgresql-contrib python3-devel tcl-devel tk-devel zlib-devel expat-devel openssl-devel -y

# ./configure

# mkdir /opt/pbs/

# ./configure --prefix=/opt/pbs/

# make

# make install

On Master

# scp /opt/pbs/etc/pbs.sh root@n001:/opt/pbs/etc/

On Node 1

# . /opt/pbs/etc/pbs.sh

# chmod +x /opt/pbs/etc/pbs.sh

# export PATH=${PATH}:/opt/pbs/bin

On Master

# scp /etc/pbs.conf root@n001:/etc/

On Node 1

# vi /etc/pbs.conf

# systemctl start pbs && systemctl enable pbs && systemctl status pbs

On Master & node2

#  yum install nfs-utils *

# systemctl start nfs && systemctl enable nfs

On Master

# chmod 777 /root/rpmbuild/RPMS/x86_64/

# vi /etc/exports

root/rpmbuild/RPMS/x86_64/ * (rw,sync,no_root_squash)

# exportfs –avr

On Node2

# mkdir /root/pbs

# mount -t nfs 192.168.44.144:/root/rpmbuild/RPMS/x86_64/ /root/pbs

# df -H

# cd pbs/

# yum install openpbs-execution-23.06.06-0.x86_64.rpm

On Node1

# rsync /etc/pbs.conf root@n002:/etc/

On Node2

# systemctl start pbs.service

# systemctl enable pbs.service

# vi /var/spool/pbs/mom_priv/config

# systemctl restart pbs.service

On Node1

# vi /var/spool/pbs/mom_priv/config

# systemctl restart pbs.service

On Master

# vi /var/spool/pbs/server_priv/nodes

n001 np=1

n002 np=1

# qmgr

Now you will get qmgr shell

# pbsnodes –a         -------> list all nodes & their attributes